# XIAO IMU Live Logger & Grapher

User Guide

Seeed Studio XIAO nRF52840 Sense

Purpose Use the XIAO as a live 6-axis IMU streamer over Bluetooth Low Energy (BLE). The Python logger saves one JSON recording approximately every 60 seconds, and the grapher turns any selected JSON file into one large PNG with accelerometer data on top and gyroscope data on the bottom.

Typical workflow

| Power XIAO | Run logger | JSON files | Choose JSON | Create PNG |
| --- | --- | --- | --- | --- |

# 1. What you need

On the XIAO side:

Seeed Studio XIAO nRF52840 Sense with the live-stream firmware already flashed.

USB-C power. Serial Monitor is optional and is used only for diagnostics.

On the host PC:

Python 3.

Bluetooth Low Energy support.

live_imu_logger.py - receives the BLE stream and saves JSON files.

imu_grapher.py - graphs one selected JSON file.

Install Python packages once Open Command Prompt or PowerShell in the folder containing the scripts, then run the command below. Bleak is used for BLE; Matplotlib and NumPy are used for graphing.

py -m pip install bleak matplotlib numpy

# 2. Recommended folder layout

Keeping the logger, grapher, recordings, and graphs together makes the commands easier to understand:

XIAO_IMU_Project\
├── host\
│   ├── live_imu_logger.py
│   └── imu_grapher.py
├── recordings\
└── graphs\

The recordings and graphs folders can be created automatically by the scripts when needed.

# 3. Start a logging session

1. Power the XIAO

Connect the XIAO to USB power. The board should begin advertising as XIAO-IMU-LIVE. You do not need to press a button or type anything into Serial Monitor.

2. Open a terminal in the host folder

Example for Windows Command Prompt or PowerShell:

cd C:\path\to\XIAO_IMU_Project\host

3. Run the BLE logger

The following command saves recordings into a folder called recordings:

py live_imu_logger.py --output-dir recordings

Expected startup The logger should print that it is scanning for 'XIAO-IMU-LIVE', connecting, subscribing to 20-byte IMU notifications, and streaming. It then keeps running until you press Ctrl+C.

# 4. How new JSON files are created

You do not manually create each JSON file. While the logger stays running, it automatically starts a new file window every 6,240 sample slots, which is approximately 60 seconds at the configured 104 Hz output data rate.

| After about 60 seconds | imu_000001_<UTC timestamp>.json |
| --- | --- |
| After about 120 seconds | imu_000002_<UTC timestamp>.json |
| After about 180 seconds | imu_000003_<UTC timestamp>.json |
| When Ctrl+C stops mid-window | imu_000004_<UTC timestamp>_partial.json |

Important: running the logger again does not overwrite the previous files because each filename includes a UTC timestamp.

# 5. Start a new experiment or make a new set of files

The cleanest approach is to use a different output folder for each experiment, subject, wearable position, or test condition.

py live_imu_logger.py --output-dir recordings\fall_test_01

py live_imu_logger.py --output-dir recordings\fall_test_02

py live_imu_logger.py --output-dir recordings\walking_test

Each command creates its own folder if it does not already exist. This keeps datasets separated without changing the logger code.

# 6. Stop logging

Press Ctrl+C in the terminal where the logger is running.

Ctrl+C

The logger stops the BLE notification subscription and, if the current one-minute window contains data, saves it as a file ending in _partial.json. A partial file is useful for inspection but is not a complete one-minute recording.

# 7/8. Graph a JSON recording

The grapher processes one JSON file at a time. You choose the exact recording you want by putting that JSON filename on the command line.

1. List the recordings

dir recordings\*.json

2. Choose one JSON filename

For example, if you want to graph imu_000003_20260818T091500.000000Z.json, run:

py imu_grapher.py recordings\imu_000003_20260818T091500.000000Z.json

3. Open the PNG

By default, the grapher writes the PNG beside the JSON using the same base filename.

recordings\imu_000003_20260818T091500.000000Z.json
recordings\imu_000003_20260818T091500.000000Z.png

# 9. What the PNG contains

Each PNG is one large image with two synchronized plots:

| Top half | Accelerometer |  |
| --- | --- | --- |
| Bottom half | Gyroscope |  |

Both plots share the same time axis. A spike at 35 seconds in the accelerometer plot lines up vertically with the gyroscope activity at 35 seconds, which makes impacts and body rotation easier to compare visually.

# Save the PNG somewhere else

Use --output when you want to choose the destination or filename yourself:

py imu_grapher.py recordings\imu_000003.json --output graphs\fall_test_03.png

# 11. Fixed scale versus automatic scale

The default graph intentionally resembles the reference fall-visualization style:

Accelerometer: -6 g to +6 g

Gyroscope: -1000 deg/s to +1000 deg/s

If the movement is small and difficult to see, use automatic scaling:

py imu_grapher.py recordings\imu_000003.json --auto-scale

# 12. Optional fall-candidate markers

The grapher can add vertical dashed markers at simple acceleration-plus-rotation threshold events. This is useful for visually finding possible fall-like impacts; it is not a validated fall-detection algorithm.

py imu_grapher.py recordings\imu_000003.json --mark-falls

Custom visualization thresholds:

py imu_grapher.py recordings\imu_000003.json --mark-falls --impact-g 2.5 --gyro-dps 250

# 13. Quick command reference

| Task | Command |
| --- | --- |
| Install packages | py -m pip install bleak matplotlib numpy |
| Start logger | py live_imu_logger.py --output-dir recordings |
| Start a new test folder | py live_imu_logger.py --output-dir recordings\fall_test_02 |
| Stop logger | Ctrl+C |
| List JSON files | dir recordings\*.json |
| Graph one file | py imu_grapher.py recordings\<filename>.json |
| Graph with auto scale | py imu_grapher.py recordings\<filename>.json --auto-scale |
| Choose PNG name | py imu_grapher.py recordings\<filename>.json --output graphs\name.png |

The logger keeps the output intentionally simple. The primary arrays are:

Timestamp

Ax, Ay, Az

Gx, Gy, Gz

SampleIndex